2b. METSTAT Model

The METSTAT solar radiation model was developed by Maxwell (1998) using quality assessed data collected from 1978 to 1980 at 29 U.S. National Weather service sites. The model calculates hourly values of direct normal, diffuse horizontal, and global horizontal solar radiation. Input parameters to the model include total and opaque cloud cover, aerosol optical depth, precipitable water vapor, ozone, surface albedo, snow depth, days-since-last-snowfall, atmospheric pressure, and present weather. The model uses deterministic algorithms to calculate accurate monthly means for each hour and statistical algorithms to simulate the statistical and stochastic characteristics of measured multiyear data sets.

The statistical algorithms randomly vary the selected input parameters of opaque cloud cover and aerosol optical depth. This simulates the effects of random hour-to-hour changes in cloud position relative to the sun and day-to-day changes in aerosol optical depth and provides estimates of hourly solar radiation with representative statistical characteristics of measured data. Without the statistical algorithms, the model would calculate the same solar radiation values whenever the input variables and the solar geometry were the same.

3a. DATSAV2 Data

DATSAV2 data consist of surface weather observations taken around the world. The primary reason for these weather observations is to support forecasting and aviation; consequently, most stations are located at airports. The observations are sent across the Global Telecommunications System (GTS), from which they are collected and decoded by the Air Force Global Weather Center (AFGWC) for forecasting purposes. AFGWC then sends the observations to the U.S. Air Force Technical Applications Center (USAFETAC) for further decoding, quality control, and archiving (Squires, 1995).

DATSAV2 data are distributed by the NCDC (Plantico and Lott, 1995) and are available as soon as three weeks after the data month. The period of record varies by station, with most beginning in 1973, but some stations have a limited amount of data as far back as the 1930s. DATSAV2 data exist for about 20,000 stations, of which 10,000 are currently active.

Most of the active stations take and transmit observations every three hours. However, some take observations every hour, while others take less frequent observations, such as every six hours or more. Some stations may not take observations during the night. Industrialized regions have more stations and observations.

DATSAV2 observations useful for solar radiation modeling and renewable energy analysis include: total sky cover, fraction of sky dome covered by low clouds, low cloud type, middle cloud type, high cloud type, ceiling, visibility, dry bulb temperature, dew point temperature, sea level pressure, wind speed, wind direction, and present weather.

4. Typical Meteorological Year Data for Selected Stations in Sri Lanka and the Maldives.

Multi-year data sets of hourly solar radiation and meteorological data were derived for selected stations using DATSAV2 data and the METSTAT model using the methodology described by Marion and George (2001). From each multi-year data set, a Typical Meteorological Year (TMY) was derived using procedures described by Marion and Urban (1995).

A TMY is a data set of hourly values of solar radiation and meteorological elements for a 1-year period. It consists of months selected from individual years and concatenated to form a complete year. The intended use is for computer simulations of solar energy conversion systems and building systems. Because of the selection criteria, these TMYs are not appropriate for simulations of wind energy conversion systems.

A TMY provides a standard for hourly data for solar radiation and other meteorological elements that permit performance comparisons of system types and configurations for one or more locations. A TMY is not necessarily a good indicator of conditions over the next year, or even the next 5 years. Rather, it represents conditions judged to be typical over a long period of time, such as 30 years. Because they represent typical rather than extreme conditions, they are not suited for designing systems and their components to meet the worst-case conditions occurring at a location.

Multi-year data sets and TMYs were developed for the locations shown in Table 4-1. City name and country follow DATSAV2 convention.

Table 4-1. Sri Lanka and Maldives Station Locations.

| WMO No. | City | Country | Time Zone | Latitude (deg min) | Longitude (deg min) | Elevation (m) |
| --- | --- | --- | --- | --- | --- | --- |
| 434000 | KANKESANTURAI_(AFB) | SB | + 6 | N 9 48 | E 80 04 | 10 |
| 434180 | TRINCOMALEE_(AFB) | SB | + 6 | N 8 35 | E 81 15 | 7 |
| 434210 | ANURADHAPURA_(AFB) | SB | + 6 | N 8 20 | E 80 25 | 89 |
| 434240 | PUTTALAM | SB | + 6 | N 8 02 | E 79 50 | 2 |
| 434360 | BATTICALOA_(AFB) | SB | + 6 | N 7 43 | E 81 42 | 12 |
| 434500 | COLOMBO/KATUNAYAKE | SB | + 6 | N 7 10 | E 79 53 | 8 |
| 434670 | COLOMBO/RATMALANA | SB | + 6 | N 6 49 | E 79 53 | 5 |
| 434730 | NUWARA_ELIYA | SB | + 6 | N 6 58 | E 80 46 | 1880 |
| 434970 | HAMBANTOTA | SB | + 6 | N 6 07 | E 81 08 | 20 |
| 435550 | MALE_INTL/HULULE_IS | MV | + 5 | N 4 12 | E 73 32 | 2 |
| 435990 | GAN_ISLAND | MV | + 5 | S 0 41 | E 73 09 | 2 |

7. References

Marion W. and George R. (2001) Calculation of solar radiation using a methodology with worldwide potential. Solar Energy 71, 4, 275-283.

Marion W. and Urban K. (1995) User’s manual for TMY2stypical meteorological years derived from the 1961-1990 National Solar Radiation Data Base. NREL/SP-463-7668. National Renewable Energy Laboratory, Golden, CO.

Maxwell E.L. (1998) METSTAT-The solar radiation model used in the production of the NSRDB. Solar Energy 62, 4, 263-279.

Plantico M.S. and Lott J.N. (1995) Foreign weather data servicing at NCDC. ASHRAE Transactions. Vol. 1, Part 1, pp. 484-490.

Squires M.F. (1995) The demographics of worldwide weather data. ASHRAE Transactions. Vol. 1, Part 1, pp. 470-475.